Condensate Traps

Condensate Drain Systems

Section   23   _ _   _ _

Download MSWord: .doc, or .pdf

Hide or display hidden notes to specifier. (Don't know how? [Click Here] )

** NOTE TO SPECIFIER ** Section   23i _  _   _  _  - CONDENSATE DRAIN SYSTEM is suggested for all HVAC condensate drains.

This section is based on products supplied by the field and maintenance provided by others.

# GENERAL

## SECTION INCLUDES

### Condensate Traps for HVAC units for Draw-Through and Blow-Through air handlers and necessary ancillary equipment for operation.

## DEFINITIONS

### Draw-Through: HVAC units in which the cooling coil (and drain pan) operate at a pressure below ambient.

### Blow-through: HVAC units in which the cooling coil (and drain pan) operate at a pressure above ambient.

## RELATED SECTIONS

### Section 23 06 30.13 - HVAC Fan Schedules

### Section 23 06 70 - Schedules for Central HVAC Equipment

### Section 23 06 80 - Schedules for Decentralized HVAC Equipment

### Section 22 05 53 - Identification for Plumbing, Piping and Equipment

### Section 22 07 19 - Plumbing Piping Insulation

### Section 22 13 16 - Sanitary Waste and Vent Piping

## REFERENCES

### ASHRAE Standard 62-89R, paragraph 5.6.4.

**NOTE TO SPECIFIER ** [Click Here]  View: Standard 62-89R

## SUBMITTALS

### Designer or manufacturer shall supply product data sheets, which define the geometry of each trap and the ancillary equipment needed for the successful operation of each air handler specified.

**NOTE TO SPECIFIER ** [Click Here] View: Example Submittal

### The designer or manufacturer shall provide detailed installation drawings, including all ancillary equipment for each air handler.

**NOTE TO SPECIFIER ** [Click Here] View: Example Drawings

### The designer or manufacturer shall provide a complete service and preventive maintenance plan, including the estimated annual cost for each air handler.

## QUALITY ASSURANCE

### Designer or Manufacturer Qualifications: Knowledge of fluid flow dynamics, including an understanding of pressures and thermal conditions inside and outside the air handler.

## DELIVERY, STORAGE AND HANDLING

### Store products in manufacturer's unopened packaging until ready for installation

# PRODUCTS

## CONDENSATE DRAIN SYSTEM

### Provide a Trapped Condensate Drain System for all HVAC air handlers.

#### Draw -Through Trap Drain System.

#### Blow -Through Trap Drain System.

### Substitutions: CostGard™ Condensate Drain Systems

**NOTE TO SPECIFIER ** [Click Here] View: CSI Specification and Drawings.

Substitutions are products of Trent Technologies located at:
15939 Old Jacksonville Highway 
Tyler, TX 75703
Tel: 903-509-4843
Email: info@TrentTech.com
Web: www.TrentTech.com  
For additional information:  [Click Here]

## ACCESSORIES

### Install field-supplied pipe supports as required by the designer or manufacturer.

### Install field-supplied pipe and fittings as required by the designer or manufacturer.

# EXECUTION

## INSTALLATION

### Allow sufficient height on the curb or slab to accommodate the drain system.

### Allow sufficient height to accommodate the drain system if suspended.

### Allow 1/8" per foot slope in drain line from drain system to condensate disposal location.

### Install in accordance with the designer's or manufacturer's instructions.

### Provide field data if requested by the designer or manufacturer.

### Contact the designer or manufacturer for technical support.

## MAINTENANCE

### Excessive

**NOTE TO SPECIFIER ** [Click Here] View: Maintenance Requirements

END OF SECTION