Condensate Drain Systems

Section   23   _ _   _ _

All links below appear in your browser.

Display hidden notes to specifier. (Don’t know how? [Click Here] )

** NOTE TO SPECIFIER ** Section   23i _  _   _  _  - CONDENSATE DRAIN SYSTEM is suggested for all HVAC condensate drains.

CostGard™ Condensate Drain Systems are products of Trent Technologies.

CostGard™ Condensate Drain Systems were developed specifically to replace the failure-prone p-trap commonly used on HVAC air handlers. They solve all the problems associated with the p-trap by eliminating destructive water damage and health-threatening conditions. Unlike the p-trap, CostGard™ Condensate Drain Systems require virtually no maintenance.

This section is based on products of Trent Technologies located at:
15939 Old Jacksonville Highway 
Tyler, TX 75703
Tel: 903-509-4843
Email: info@TrentTech.com
Web: www.TrentTech.com

# GENERAL

## SECTION INCLUDES

### CostGard™ Condensate Drain System for HVAC units for Draw-Through and Blow-Through air handlers.

## DEFINITIONS

### Draw-Through: HVAC units where the cooling coil (and drain pan) operates at a pressure below ambient.

### Blow-Through: HVAC units where the cooling coil (and drain pan) operates at a pressure above ambient.

## RELATED SECTIONS

### Section 23 06 30.13 - HVAC Fan Schedules

### Section 23 06 70 - Schedules for Central HVAC Equipment

### Section 23 06 80 - Schedules for Decentralized HVAC Equipment

### Section 22 05 53 - Identification for Plumbing, Piping and Equipment

### Section 22 07 19 - Plumbing Piping Insulation

### Section 22 13 16 - Sanitary Waste and Vent Piping

## REFERENCES

### McGraw-Hill: HVAC Handbook, 2004, Chapter 18.8

**NOTE TO SPECIFIER ** [Click Here]  View: HVAC Handbook

### ASHRAE Standard 62-89R, paragraph 5.6.4.

**NOTE TO SPECIFIER ** [Click Here]  View: Standard 62-89R

## SUBMITTALS

### Manufacturer shall supply product data sheets on each product to be used.

### Manufacturer shall provide installation drawings.

## QUALITY ASSURANCE

### Manufacturer Qualifications: Single source for design, engineering, production, and technical support.

### Manufacturers shall have at least 20 years of experience in manufacturing condensate drain systems.

## DELIVERY, STORAGE AND HANDLING

### Store products in the manufacturer's unopened packaging until ready for installation.

# PRODUCTS

## CONDENSATE DRAIN SYSTEM

### Provide CostGard™ Condensate Drain System for all HVAC air handlers.

#### Draw-Through CostGard™ Drain System.

**NOTE TO SPECIFIER ** [Click Here]  View: Draw-Through Submittal

#### Blow-Through CostGard™ Drain System.

**NOTE TO SPECIFIER ** [Click Here]  View: Blow-Through Submittal

### Acceptable Manufacturer: Trent Technologies located at 15939 Old Jacksonville Highway, Tyler, Texas 75703; Tel: 903-509-4843; Email: info@TrentTech.com; Web: www.TrentTech.com.

#### Provide HVAC submittals and equipment schedules to Trent Technologies.

### Substitutions: Not permitted.

## ACCESSORIES

### Install field-supplied pipe supports as required by the manufacturer.

### Install field-supplied pipe and fittings as required by the manufacturer.

# EXECUTION

## INSTALLATION

### Allow sufficient height on the curb or slab to accommodate the drain system.

### Allow sufficient height to accommodate the drain system if suspended.

### Allow 1/8" per foot slope in drain line from drain system to condensate disposal location.

### Install in accordance with the manufacturer's instructions.

### Provide field data if requested by the manufacturer.

### Test and balance for proper operation. Adjust and retest until proper operation is achieved in accordance with the manufacturer's instructions.

### Contact the manufacturer for technical support.

## MAINTENANCE

### None

END OF SECTION